# SECTION 23 3400

# HVAC Fans

## PART 1  GENERAL

1.1	DESCRIPTION

A.	Provide Fans as specified herein and shown on the Drawings.

B.	Equipment capacity and size as indicated in the equipment lists on the Drawings.

C.	Related Work:  The requirements of Section 23 0500, Common HVAC Materials and Methods, also apply to this section.

1.2	QUALITY ASSURANCE

A.	Air Handling Equipment:  Rated in accordance with AMCA certified rating procedures and AMCA labeled.

1.3	SUBMITTALS

A.	Submit catalog data, construction details and performance characteristics for each fan.

B.	Submit operating and maintenance data.

## PART 2  PRODUCTS

  - Sidewall exhaust fans

- Constructed for sidewall mounting, direct driven, open BI wheel as shown on Drawings with pressure lubricated ball bearings; ball bearing fan duty motor; vibration isolated; bird screen; weatherproof aluminum housing; capacity as indicated on Drawings.  Motor located outside the air stream.  Casing to be easily removed for service.  Motor and fan assembly to be mounted on rubber vibration isolators. Where indicated on the Drawings, provide backdraft damper.  Greenheck , Loren Cook, Twin City ..

  - EXHAUST FANS  (UL listed for Restaurant Greast Exhaust (UL/cUL-762 ) for GEF-1 & GEF-2.)

- Grease Rated Utility Set Exhaust Fan: UL 762 listed for outdoor installation with welded wheel construction. AMCA rated B.I. wheetl capacity as schedule on Drawings. Mount motor on adjustable base; pressure gun lubricated ball or roller bearing for both fan and motor. Greenheck USF series, or Loren Cook, Twin City.

## PART 3  EXECUTION

3.1	INSTALLATION

A.	Install and arrange equipment as shown on the Drawings and as recommended by the equipment manufacturer.

3.3	AIR HANDLING INSTALLATION

A.	Installation and Arrangement:  Air handling equipment shall be installed and arranged as shown on the Drawings.  Comply with the manufacturer's recommendations for installation connection and start-up.

B.	Lubrication:  All moving and rotating parts shall be lubricated in accordance with the manufacturer's recommendations prior to start-up.

C.	Filters:  Specified filters or approved temporary construction filters shall be installed in supply units prior to start-up or used for drying and/or temporary heat.

3.4	CONTROLS

A.	Wiring:  All wiring shall be in accordance with the National Electrical Code and local electrical codes.

END OF SECTION 23 3400