# SECTION 22 40 00 - PLUMBING fixtures

## PART  1   GENERAL

1.01		DESCRIPTION

A.	The requirements of this section apply to the plumbing fixtures and trim.

B.	Provide fixtures as shown on the Drawings and specified herein.  Provide all required fixture trim and accessories for a complete, finished installation.

C.	Related Work:  The requirements of Section 22 05 00, Common Plumbing Materials and Methods, also apply to this section.

1.02		QUALITY ASSURANCE

A.	Code:  Comply with requirements of the Idaho State Plumbing Specialty Code.

B.	Fixture color: White unless indicated otherwise.

C.	Fixture Samples:  Provide actual samples of plumbing fixtures and faucets prior to or included with the submittal package.  Approved samples may be incorporated into the project.

D.	Potable Water Valves:  Potable water valves not limited to faucets, mixing valves, or pressure reducing valves.  Valves shall meet NSF Standard 61, Section 9, for drinking water faucets and shall be brass construction.  Brass components which contact water within the faucet shall be from brass which contains no more than 3 percent lead by dry weight.

## PART  2   PRODUCTS

2.01		PIPING

A.	Piping, fittings, and related items as specified in related Sections 22 10 00.

2.02		PLUMBING FIXTURES and trim

A.	Stops:  Furnish stop valves for all fixtures.  Quarter turn ball valve style, in wall, angle or straight through pattern to fit installation.   Compression nuts to be high copper content brass.  Finish to be copper nickel chrome plate where concealed. Finish on exposed stops as noted. Product to carry manufacturer’s name.   Provide chrome plated shallow escutcheons.  McGuire, Chicago, Brasskraft, Keeney, Zurn, or approved substitute.

B.	Fixture Traps:  Exposed fixture tailpieces, traps, and wastes shall be chrome plated 17 gauge seamless brass tube with deep or box style escutcheons as required to conceal rough piping.  Products to be stamped with manufacturer’s name and material gauge.  McGuire, Keeney, Zurn, or approved.  Concealed fixture traps may be rigid plastic.

C.	Provide handicap piping protector kit on all exposed accessible fixture traps and supplies (I&S Insulation Co. Inc., Brocar Products Inc. kit 500R, McGuire “Prowrap,” Plumberex “Pro-2000” or approved substitute.

D.	Water Closets 1.28 GPF, Water Closet, Elongated Type, Vitreous China:  Floor mounted two piece close coupled, gravity tank type water closet shall be specifically designed for 1.28 gallon siphon jet flush.  Minimum MaP score of 1000.  Provide satin nickel finish stop, escutcheon, and flush lever.  Public Restroom ADA water closet:  Same as above.

1.	“WC-1”: Residential, Dwelling Unit Water Closet Toto Drake II

a.	Manufacturer: Toto

b.	Model: Drake II 1G – CST454CUFG#01

c.	Finish: #01 Cotton

d.	Residential ADA Units:  Same as above.

1) 	Seat:  White molded wood seat, with bumpers; closed front with cover to match bowl. Bemis 400.00.  Church, Olsonite approved. Color #01 Cotton.

2.	“WC-2”:  Public-ADA

a.	Manufacturer: Toto

b.	Model: Drake II 1G – CST454CUFG#01

c.	Finish: #01 Cotton

1)	Seat:  White molded wood seat, with bumpers; closed front with cover to match bowl. Bemis 400.00.  Church, Olsonite approved. Color #01 Cotton.

E.	Lavatory (Residential) LV-1:

1.	Faucet:

a.	Manufacturer:  Grohe

b.	Model: 34271001-Chrome finish 001

c.	Finish: Stainless Steel

3.	Sink: Kohler

b.	Model: Verticyl

a.	Manufacturer:

c.	Finish: White (020)

F.	Lavatory LV-3 (Public):

1.	Faucet Public:

a.	Manufacturer: Grohe

b.    Model: Euroeco cosmopolitan E 3627 1000

c.	Finish: Brushed Stainless

2.	Sink:

a.	Manufacturer: Kohler

b.	Model: Soho K-2084-0

c.	Finish: White

H.	Kitchen Sink KS-1 (Type B):

1.	Faucet:

a.	Manufacturer: Grohe

b.	Model: tbd

c.	Finish: Stainless Steel

2.	Bowl:

a.	Manufacturer: Moen

b.	Model: GS18191

I.	Kitchen Sink KS-2 (Type A, L/W, Leasing Kitchen):

1.	Faucet:

a.	Manufacturer: Pfister

b.	Model: Soho LG529-SAS

c.	Finish: Stainless Steel

2.	Bowl:

a.	Manufacturer: Moen

b.	Model: GS18195B – shallow ADA Version

J.	Roof Deck Sink KS-3

1.	Faucet:

a.	Manufacturer: Pfister

b.	Model: Soho LG529-SAS

c.	Finish: Stainless Steel

2.	Bowl:

a.	Manufacturer: Elkay

b.	Model: ELUHAD211555PD

K.	Community Room Sink KS-4:

1.	Faucet:

a.	Manufacturer: Moen

b.	Model: 5965BL

c.	Finish: Matte Black

2.	Bowl:

a.	Manufacturer: Elkay

b.	Model: ELGU2522

c.	Finish: Black

3.	Drain Fitting:

a.	Manufacturer: Elkay

b.	Model: LKQS35BK

c.	Finish: Black

L.	“TS-1” Tub / Shower:

* See Architectural plans for exact type and location.

(See Architect Reference B-1 through B-4):

M.	“B-1” Bath Surround:  (Type A & B units) (or approved equal)

1.	Manufacturer: Fiber Care Baths

2.	Model: WT60-30HC L/R

a.	Faucet:

1)	Manufacturer: Delta

2)	Model: T14459-SSLHD

b.	Finish: Stainless (faucet head)

1)	Manufacturer: Delta

2)	Model: RP70175SS

3)	Finish: Stainless

N.	“B-2” Shower Pan (w/ tile surround):   (Type B Units only) (or approved equal)

1.	Manufacturer: Fiber Care Baths

2.	Model: WT60-30HC L/R

a.	Faucet:

1)	Manufacturer: Delta

2)	Model: T14459-SSLHD

3)	Finish: Stainless

b.	Faucet head

1)	Manufacturer: Delta

2)	Model: RP70175SS

3)	Finish: Stainless

O.	“B-3” L/W Shower: This will be a fully tiled shower (floor and walls) with built-in grab bars and a shower seat.  Type A & Accessible Faucet:

1.	Faucet:

a.	Manufacturer: Delta

b.	Model: T14459-SSLHD

c.	Finish: Stainless

2.	Faucet Head / Slide Bar:

a.	Manufacturer: Delta

b.	Model: 51361-SS

c.	Finish: Stainless

Q.	“DF-1” Drinking Fountain:

1.	Manufacturer: Elkay

2.	Model: EZWS-EDFPBM117K

R.	“MS-1” Mop Sink:

1.	Basin:  Mustee 63M or approved equal.

2.	Faucet:  Chicago 897-MPRCF or approved equal.

S.	“BW-1” Bike Wash:

1.	Faucet:  HB-2 or approved equal:

Cast aluminum / stainless steel wall box with wall flange, hold open cast door, vacuum breaker, stop, single temperature Acorn 8104 or approved.

T.	“PT-1” Pet Tub

a.	Tub:  Groomers Best, Inc. GB58E, ADA.

b.	Faucet:  T & S Brass B-0167.

U.	Clothes Washer Fitting, CW-1: Guy Gray WB 200/T 200 hot and cold water valve washing machine waste outlet.

1.	Refrigerator Water Connection, RC-1:  Guy Gray BIM 875 or approved.

V.	Hose Bibs:

1.	Outside "HB-1":  Non-freeze type with vacuum breaker, bronze wall casing and wall clamp.  Zurn Z-1310-6, Wade W-8620, Woodford 67 series, Smith 5609-PB, or Watts HY420.

2.	Inside "HB-2":  Cast aluminum / stainless steel wall box with wall flange, hold open cast door, vacuum breaker, stop, single temperature Acorn 8104 or approved.

3.	“HB-3”:  Sillcock with vacuum breaker Woodford model 26 or equal.

W.		Floor Sinks/Floor Drains “FD-1/FS-1” : Zurn, Jay R. Smith, Wade and Mifab.  Numbers scheduled on drawings represent minimum acceptable standard for locations involved.

## PART 3   EXECUTION

3.01		PIPING

A.	Install in accordance with Section 22 10 00.

B.	Hot and cold water stubouts to fixtures shall be copper.  Secure stubouts with manufactured bracket system.  Hubbard “Holdrite”, Sioux chief, or approved.

3.02		FIXTURE INSTALLATION AND CONNECTION

A.	All exposed fixture hardware and piping shall be plated with polished chrome unless otherwise directed in these specifications.  Where chair carriers or special carrier design are not indicated, provide wall blocking for anchorage of manufacturers standard mounting bracket.

B.	All fixtures in contact with finished walls and floors shall be caulked with waterproof, white, non-hardening sealant which will not crack, shrink or change color with age.

C.	All fixtures and component parts shall conform to governing codes.

D.	All fixtures shall be securely mounted level and plumb or as recommended by the manufacturer.  Mount fixtures intended to be accessible to the handicapped at the dimensions required by code.

3.03		startup

A.	Adjust flush valves, pressure reducing valves, mixing valves, water heater thermostats, and similar equipment.

B.	Remove construction protection, tags and labels and thoroughly clean all plumbing equipment and trim. Scour all fixtures just prior to building acceptance.

END OF SECTION

THIS PAGE INTENTIONALLY BLANK