# SECTION 23 0550 – MECHANICAL DEMOLITION

## PART 1 - GENERAL

1.1	RELATED DOCUMENTS

A.	This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

Retain or delete this article in all Sections of Project Manual.

1.2	SUMMARY

A.	Section Includes:

1.	Revise subparagraphs below to suit Project.

2.	Demolition and removal of selected components of mechanical systems.

3.	Cutting and patching of building assemblies as required for mechanical demolition.

Temporary partitions.

1.3	DEFINITIONS

A.	Remove:  Detach items from existing construction and legally dispose of them off-site unless indicated to be removed and salvaged or removed and reinstalled.

B.	Remove and Salvage:  Carefully detach from existing construction, in a manner to prevent damage, and deliver to Owner where indicated on drawings.

C.	Remove and Reinstall:  Detach items from existing construction, prepare for reuse, and reinstall where indicated.

D.	Existing to Remain:  Existing items of construction that are not to be permanently removed and that are not otherwise indicated to be removed, removed and salvaged, or removed and reinstalled.

E.	Cutting:  Removal of in-place construction necessary to permit installation or performance of other work.

F.	Patching:  Fitting and repair work required to restore construction to original conditions after installation of other work.

1.4	MATERIALS OWNERSHIP

A.	Unless otherwise indicated, demolition waste becomes property of Contractor.

1.5	INFORMATION SUBMITTALS

A.	Coordinate "Qualification Data" Paragraph below with qualification requirements in Section 01 4000 "Quality Requirements" and as may be supplemented in "Quality Assurance" Article.

B.	Retain "Proposed Protection Measures" Paragraph below if selective demolition operations occur adjacent to occupied spaces and an informational submittal is required.

C.	Proposed Protection Measures:  Submit report, including drawings, that indicates the measures proposed for protecting individuals and property, for environmental protection, for dust control and, for noise control.  Indicate proposed locations and construction of barriers.

D.	"Schedule of Selective Demolition Activities" Paragraph below may be used to track Contractor's progress; it may also be used to determine that selective demolition will not interfere with Owner's operations.

E.	Schedule of Selective Demolition Activities:  Indicate the following:

1.	Detailed sequence of selective demolition and removal work, with starting and ending dates for each activity.  Ensure Owner's building manager's and other tenants' on-site operations are uninterrupted.

2.	Interruption of utility services.  Indicate how long utility services will be interrupted.

3.	Coordination for shutoff, capping, and continuation of utility services.

4.	Coordination of Owner's continuing occupancy of portions of existing building and of Owner's partial occupancy of completed Work.

F.	Inventory:  Submit a list of items to be removed and salvaged and deliver to Owner prior to start of demolition.

G.	Statement of Refrigerant Recovery:  Signed by refrigerant recovery technician responsible for recovering refrigerant, stating that all refrigerant that was present was recovered and that recovery was performed according to EPA regulations.  Include name and address of technician and date refrigerant was recovered. Also see mechanical specifications for further requirements.

1.6	QUALITY ASSURANCE

A.	Refrigerant Recovery Technician Qualifications:  Certified by an EPA-approved certification program.

1.7	FIELD CONDITIONS

A.	Retain, revise, or delete this article to suit Project.  Insert other limitations if necessary, such as when adjacent floors will be occupied.

B.	Owner will occupy portions of building immediately adjacent to selective demolition area.  Conduct selective demolition so Owner's operations will not be disrupted.

C.	Review first paragraph below and revise if necessary.  In subparagraph, include list of items that will be removed by Owner.

D.	Conditions existing at time of inspection for bidding purpose will be maintained by Owner as far as practical.

E.	Notify Engineer of discrepancies between existing conditions and Drawings before proceeding with selective demolition.

F.	Retain one of two "Hazardous Materials" paragraphs below, or remove all references to hazardous materials.  Insert scope of article to include asbestos, PCBs, and other materials if required.  Coordinate statements with the General and Supplementary Conditions.  See "Hazardous Materials" Article in the Evaluations.

G.	Hazardous Materials:  Hazardous materials may be encountered in the Work.

1.	Hazardous materials will be removed by Owner before start of the Work.

2.	If suspected hazardous materials are encountered, do not disturb; immediately notify Engineer and Owner.  Hazardous materials will be removed by Owner under a separate contract.

H.	Retain "Historic Areas" Paragraph below if historic removal or dismantling is required.

I.	Movement of materials:  Demolition and hauling equipment and other materials shall be of sizes that clear surfaces within spaces, areas, rooms, and openings, including temporary protection, by 6 inches or more.

J.	Storage or sale of removed items or materials on-site is not permitted.

K.	Utility Service:  Maintain existing utilities indicated to remain in service and protect them against damage during selective demolition operations.

1.	Maintain fire-protection facilities in service during selective demolition operations.

1.8	WARRANTY

A.	Existing Warranties:  Remove, replace, patch, and repair materials and surfaces cut or damaged during selective demolition, by methods and with materials so as not to void existing warranties.  Notify warrantor before proceeding.  Existing warranties may include the following:

1.	Roofing systems.

B.	Notify warrantor on completion of selective demolition, and obtain documentation verifying that existing system has been inspected and warranty remains in effect.  Submit documentation at Project closeout.

## PART 2 - PRODUCTS

1.2	PEFORMANCE REQUIREMENTS

A.	Regulatory Requirements:  Comply with governing EPA notification regulations before beginning selective demolition.  Comply with hauling and disposal regulations of authorities having jurisdiction.

B.	Standards:  Comply with ANSI/ASSE A10.6 and NFPA 241.

## PART 3 - EXECUTION

3.1	EXAMINATION

A.	Verify that utilities have been disconnected and capped before starting selective demolition operations.

B.	Review record documents of existing construction provided by Owner.  Owner does not guarantee that existing conditions are same as those indicated on existing drawings.

C.	Survey existing conditions and correlate with requirements indicated to determine extent of selective demolition required.

D.	When unanticipated mechanical, electrical, or structural elements that conflict with intended function or design are encountered, investigate and measure the nature and extent of conflict.  Promptly submit a written report to Engineer and Owner.

E.	Survey of Existing Conditions:  Record existing conditions by use of measurements, preconstruction photographs and templates.

1.	Before selective demolition or removal of existing building elements that will be reproduced or duplicated in final Work, make permanent record of measurements, materials, and construction details required to make exact reproduction.

3.2	UTILITY SERVICES AND MECHANICAL/ELECTRICAL SYSTEMS

A.	See Mechanical, Plumbing, and Electrical specifications for utility services and mechanical/electrical system requirements.

B.	Refrigerant:  Remove refrigerant from mechanical equipment to be selectively demolished according to 40 CFR 82 and regulations of authorities having jurisdiction.

3.3	PREPARATION

A.	Site Access and Temporary Controls:  Conduct selective demolition and debris-removal operations to ensure minimum interference with roads, streets, walks, walkways, and other adjacent occupied and used facilities.

B.	Temporary Facilities:  Provide temporary barricades and other protection required to prevent injury to people and damage to adjacent buildings and facilities to remain.

1.	Provide protection to ensure safe passage of people around selective demolition area and to and from occupied portions of building.

2.	Provide temporary weather protection, during interval between selective demolition of existing construction on exterior surfaces and new construction, to prevent water leakage and damage to structure and interior areas.

3.	Protect walls, ceilings, floors, and other existing finish work that are to remain or that are exposed during selective demolition operations.

4.	Cover and protect furniture, furnishings, and equipment that have not been removed.

C.	Temporary Shoring:  Provide and maintain shoring, bracing, and structural supports as required to preserve stability and prevent movement, settlement, or collapse of construction and finishes to remain, and to prevent unexpected or uncontrolled movement or collapse of construction being demolished.

1.	Strengthen or add new supports when required during progress of selective demolition.

3.4	SELECTIVE DEMOLITION, GENERAL

A.	General:  Demolish and remove existing construction only to the extent required by new construction and as indicated.  Use methods required to complete the Work within limitations of governing regulations and as follows:

1.	Proceed with selective demolition systematically, from higher to lower level.  Complete selective demolition operations above each floor or tier before disturbing supporting members on the next lower level.

2.	Neatly cut openings and holes plumb, square, and true to dimensions required.  Use cutting methods least likely to damage construction to remain or adjoining construction.  Use hand tools or small power tools designed for sawing or grinding, not hammering and chopping, to minimize disturbance of adjacent surfaces.  Temporarily cover openings to remain.

3.	Cut or drill from the exposed or finished side into concealed surfaces to avoid marring existing finished surfaces.

4.	Do not use cutting torches until work area is cleared of flammable materials.  At concealed spaces, such as duct and pipe interiors, verify condition and contents of hidden space before starting flame-cutting operations.  Maintain fire watch and portable fire-suppression devices during flame-cutting operations.

5.	Maintain adequate ventilation when using cutting torches.

6.	Remove decayed, vermin-infested, or otherwise dangerous or unsuitable materials and promptly dispose of off-site.

7.	Locate selective demolition equipment and remove debris and materials so as not to impose excessive loads on supporting walls, floors, or framing.

8.	Dispose of demolished items and materials promptly.

B.	Reuse of Building Elements:  Do not demolish building elements beyond what is indicated on Drawings without Engineer's approval.

3.5	DISPOSAL OF DEMOLISHED MATERIALS

A.	First option in "Nonshell Elements" Subparagraph below is requirement for LEED-CI Credit MR 1.2; second option is for Credit MR 1.3.

B.	General:  Except for items or materials indicated to be reused, salvaged, reinstalled, or otherwise indicated to remain Owner's property, remove demolished materials from Project site and legally dispose of them in an EPA-approved landfill.

1.	Do not allow demolished materials to accumulate on-site.

2.	Remove and transport debris in a manner that will prevent spillage on adjacent surfaces and areas.

3.	Remove debris from elevated portions of building by chute, hoist, or other device that will convey debris to grade level in a controlled descent.

C.	Burning:  Do not burn demolished materials.

D.	Disposal:  Transport demolished materials off Owner's property and legally dispose of them.

3.6	CUTTING AND PATCHING

A.	Temporary Support:  Provide temporary support of work to be cut.

B.	Protection:  Protect in-place construction during cutting and patching to prevent damage.  Provide protection from adverse weather conditions for portions of Project that might be exposed during cutting and patching operations.

C.	Adjacent Occupied Areas:  Where interference with use of adjoining areas or interruption of free passage to adjoining areas is unavoidable, coordinate cutting and patching as required.

D.	Existing Utility Services and Mechanical/Electrical Systems:  Where existing services/systems are required to be removed, relocated, or abandoned, bypass such services/systems before cutting to minimize and prevent interruption to occupied areas.

3.7	TEMPORARY PARTITIONS

A.	Provide floor-to-ceiling dustproof partitions to limit dust and dirt migration and to separate areas occupied by Owner and tenants from fumes and noise.

1.	Construct dustproof partitions with two layers of 6-mil polyethylene sheet on each side.  Cover finished floors with two layers of 6-mil polyethylene sheet, extending sheets 18 inches up the sidewalls.  Overlap and tape full length of joints.  Cover finished floors with fire-retardant-treated plywood.

2.	Where fire-resistance-rated temporary partitions are indicated or are required by authorities having jurisdiction, construct partitions according to the rated assemblies.

3.	Protect air-handling equipment.

4.	Provide walk-off mats at each entrance through temporary partition.

3.8	CLEANING

A.	Clean adjacent structures and improvements of dust, dirt, and debris caused by selective demolition operations.  Return adjacent areas to condition existing before selective demolition operations began.

END OF SECTION