# SECTION 26 0920

# LIGHTING CONTROL system

## PART 1 GENERAL

1.01  	INTRODUCTION

A.	The provisions of the General Requirements, Supplementary Requirements, and Division 1 		apply to the electrical work specified in this Section.

B.	Contractor shall coordinate all work in this section with other sections and divisions of these 	specifications to provide a complete and operable system.

1.02  	DESCRIPTION

A.   	Extent of lighting control system work is indicated by drawings and by the requirements of this 	section.

B. 	Type of lighting control equipment and wiring specified in this section include, but are not limited to, the following:  Microprocessor based processor panel, dimming panels, and switching panels.

C.  	Requirements are indicated elsewhere in these specifications for work including, but not limited to, raceways and electrical boxes and fittings required for installation of control equipment and wiring.

1.03  	QUALITY ASSURANCE

A. 	The control panels shall be tested and listed under the UL 924 Energy Management Equipment 	standards and CSA standards.

B. 	The control system shall comply with all applicable National Electrical Codes regarding electrical 	wiring standards.

C. 	All control equipment shall undergo strict inspection standards.  The equipment shall be previously tested and burned-in at the factory prior to installation.

D. 	A factory-trained technician or factory-authorized personnel shall assist in installation and 	programming, functionally test the control system and verify performance after installation.

E. 	Approved manufacturers:  Lutron per landlord requirements – no substitutes.

1.04 	SUBMITTALS

A. 	Submit manufacturer’s data on lighting control system and components.

B. 	Submit drawings of lighting control system and accessories including, but not limited to, panels, 	switches, photocells and other associated devices and equipment.

## PART 2 PRODUCTS

2.01	MATERIALS AND COMPONENTS

A.	Processor Panel:

1.	GRAFIK 6000 Processor Panel.

B.	Dimming Panels:

1.	GP Dimming Panel.

C.	Switching Panels:

1.		XP Softswitch Panel.

## PART 3 EXECUTION

3.01	EQUIPMENT INSTALLATION AND DOCUMENTATION

A. 	Installation:  The control panels and equipment shall be installed, connected and fully wired 		as shown on the shop drawings.

B. 	Documentation:  The contractor shall provide record drawings to the owner for correct 			programming and proper maintenance of the control panels. The record drawings shall 			indicate the load controlled by each relay or dimmer and the relay or dimmer panel number.

C. 	Operation and Service Manuals:  Upon completion of Contract submit Operating and 			Maintenance Manuals for inclusion in Owner's Maintenance Brochure as specified in Division 		1.

3.02  	PRODUCT SUPPORT AND SERVICE

A.	Factory Support:  Factory telephone support shall be available at no cost to the owner. Factory 	assistance shall consist of solving programming or application questions concerning the control 	equipment.

3.03  	SYSTEM ACCEPTANCE

A.	The contractor is responsible for complete installation of the system according to factory standards and requirements.  The following items shall constitute factory standards and requirements:

1.	All system equipment shall operate in accordance with specifications.

2.	An operational user program shall exist in the control system. The program shall execute and perform all functions required to effectively operate the site according to the requirements.

3.	Demonstration of program integrity during normal operation and pursuant to a power outage.

4.	Contractor shall provide minimum four hours of training on the operation and use of the 	control system. Additional support services shall be negotiated between the contractor and the building owner or manager.

3.04 	WARRANTY

A. 	Manufacturer shall supply a 3-year warranty on all hardware and software. A limited 10-year 		warranty shall be provided on the individual relays.

END OF SECTION